TuneHUD + Balance Bot

Full Project Handoff Document

Closed Loop Systems — Ryan Lush

# 1. Business Context

Company: Closed Loop Systems (domain pending — closedloopsystems.com taken, looking at alternatives)

Product: TuneHUD — live PID tuning and instrumentation platform

Domain target: tunehud.io

GitHub: lusher00 (personal), closedloopsystems (org, pending)

Business model: Open source everything, revenue from hosted gateway SaaS, custom development, OEM licensing. ESP32 library always free as the funnel.

Strategy: Build the "tuner" identity for industrial controls — same cultural cachet as automotive tuning but for industrial systems. Hackaday article is the launch vehicle. Demo video is the most important near-term deliverable.

# 2. TuneHUD Architecture

## 2.1 Overview

TuneHUD is a transport-agnostic control system tuning and instrumentation platform. A gateway process runs on a Pi 5 or any Linux host, connects to one or more controllers via various transports, and serves a live dashboard to any browser or phone over WebSocket.

Key design decisions:

- Single WebSocket port (8765) serves both WebSocket connections and dashboard HTML (via process_request)

- MJPEG camera stream on separate port 8769 via Flask in a background thread

- Devices auto-register by sending manifest_resp on connect — no config needed for ESP32

- Multi-controller: each controller namespaced as ctrl.param in stream_data messages

- Cluster builder lets user configure gauge types (numeric, arc, bar, vbar) per controller at runtime

## 2.2 Gateway — main.py

Location: ~/TuneHUD/main.py on Pi 5 (192.168.1.126)

Key classes and methods:

- TuneHUDGateway — main class, owns plugins, clients, camera, session logger

- _process_request — serves dashboard HTML for HTTP GET, passes WebSocket upgrades through

- _ws_handler — handles all WebSocket connections (both dashboard clients and device clients)

- _handle_message — routes manifest_resp (device registration), stream_data (device forwarding), param_write, manifest_req, ping

- _stream_loop — polls all configured transport plugins at stream_hz, broadcasts stream_data per controller

- _connect_loop — reconnects dropped transport plugins every 5 seconds

- _start_mjpeg_flask — starts Flask MJPEG server on port 8769 in daemon thread

- _device_clients — dict of id(ws) -> {ws, name, latest} for auto-registered devices

## 2.3 Transport Plugins

All plugins in ~/TuneHUD/plugins/. Each implements TransportPlugin base class:

- websocket.py — outbound WebSocket client (connects to demo nodes, future ESP32 devices in server mode)

- modbus_tcp.py — Modbus TCP client

- modbus_rtu.py — Modbus RTU over serial

- serial_transport.py — raw serial

- ble_transport.py — BLE peripheral via bleak (scan by name or MAC)

- odrive_transport.py — ODrive 3.6 / ODesc 3.6 clone via USB using odrive Python package

## 2.4 Config Format

# configs/multi_demo.yaml

title: "Multi-Controller Demo"

server:

host: 0.0.0.0

port: 8765

serve_dashboard: true

stream:

max_hz: 100

default_hz: 20

controllers:

- name: velocity

type: websocket

host: 127.0.0.1

port: 8766

gauges:

- param: pv

type: arc

min: -100

max: 100

## 2.5 WebSocket Protocol

Dashboard → Gateway:

- {"type":"manifest_req"} — request combined manifest

- {"type":"stream_start","hz":20} — set stream rate

- {"type":"param_write","controller":"velocity","name":"kp","value":1.5} — write param

- {"type":"ping"} — keepalive

Gateway → Dashboard:

- {"type":"manifest_resp","title":"...","controllers":[{name,device,transport,params[],gauges[]}]} — full manifest

- {"type":"stream_data","controller":"velocity","t":1234.5,"data":{"kp":1.5,...}} — live data

- {"type":"write_ack","controller":"velocity","name":"kp","value":1.5,"t":...} — write confirmation

Device → Gateway (auto-registration):

- Device sends manifest_resp on connect — gateway detects and registers as dynamic controller

- Device sends stream_data — gateway forwards to all dashboard clients

## 2.6 Dashboard — tunehud_dashboard.html

Single file, served by gateway at http://192.168.1.126:8765/

Tabs: Overview, Tune, Graph, Cluster Builder, Session Log, Debug, Settings

Key features:

- Overview: gauge clusters per controller (numeric/arc/bar/vbar), camera feed from port 8769

- Tune: sliders send on release (onchange not oninput), step buttons, val boxes

- Graph: multi-plot, multi-series, 5-120s window, CSV recording, default plots prefer pv/setpoint/error/output

- Cluster Builder: add/remove/configure gauges per controller at runtime

- Debug: separate section per controller, updates in place

- Rate display: rolling 2s window packet count, integer Hz

## 2.7 Camera

camera.py uses rpicam-vid subprocess — exact same command as hailo-cat-tracker for correct autofocus:

rpicam-vid --codec mjpeg --inline --nopreview --width 1920 --height 1080

--framerate 30 --denoise off --autofocus-mode manual --flush --timeout 0 --output -

Frame parsing: reads 16384-byte chunks, finds JPEG SOI/EOI markers, stores latest frame in lock.

Flask serves MJPEG on port 8769 — browser connects once, frames flow continuously.

Camera MJPEG was flickering with asyncio TCP server — switched to Flask in daemon thread. This is the working approach.

## 2.8 Running the Demo

cd ~/TuneHUD

./start.sh                    # kills old procs, starts demo nodes, starts gateway

# or manually:

pkill -f demo_node; pkill -f "python main.py"; pkill -f rpicam-vid; sleep 2

python demo_node.py &

sleep 1 && python demo_node.py --port 8767 &

sleep 1 && python main.py --config configs/multi_demo.yaml

Dashboard: http://192.168.1.126:8765

Camera: http://192.168.1.126:8769/camera

# 3. TuneHUD ESP32 Arduino Library

## 3.1 Overview

Library: TuneHUD_ESP32 — Arduino library for ESP32 devices

Location: ~/Documents/Arduino/libraries/TuneHUD_ESP32/

GitHub target: github.com/lusher00/TuneHUD_ESP32

Dependencies: WebSockets by Markus Sattler, ArduinoJson by Benoit Blanchon

## 3.2 API

TuneHUD hud;

hud.begin("My Controller", 20);          // device name, stream Hz

hud.addParam("kp", &kp, 0.0, 10.0, 0.01, "PID gains");

hud.addReadOnly("pv", &pv, "Control", "rpm");

hud.connectWiFi("192.168.1.126", 8765);  // gateway IP

hud.enableBLE();                          // advertise as TuneHUD-<name>

hud.onWrite(callback);                    // fired after variable updated

hud.update();                             // call every loop()

## 3.3 BLE

ESP32 runs WiFi WebSocket + BLE peripheral simultaneously. BLE GATT service:

- Manifest characteristic (read) — returns manifest JSON

- Stream characteristic (notify) — sends stream_data JSON on each update

- Write characteristic (write) — receives param_write JSON from phone/gateway

Gateway BLE plugin (bleak) scans by scan_name or connects by MAC address.

## 3.4 Device Auto-Registration

ESP32 sends manifest_resp on WebSocket connect. Gateway detects this and registers it as a dynamic controller — no config YAML needed. The Feather shows up automatically as "feather_motor_pid" (sanitized device name).

## 3.5 Examples

- BasicPID — simulated first-order plant, good starting point

- TemperatureControl — thermistor + PWM heater

- CascadePosition — outer position loop / inner velocity loop, WiFi+BLE

# 4. Balance Bot

## 4.1 Hardware

Board: BeagleBone Blue (boneblue-0, debian@192.168.4.x)

Motor controller: RoboClaw 2x15A, addr 0x80, /dev/ttyO1, 460800 baud

Motors: RS-555, 5.2:1 gearbox, 145.1 PPR encoders, 155mm wheels

IMU: MPU-9250 onboard, custom DMP driver (no librobotcontrol)

OLED: SSD1306 128x64, I2C bus 2, addr 0x3C

All polarities: motor_pol=-1.0, enc_pol=-1.0

## 4.2 Control Architecture

Cascade PID:

- D1 — balance/pitch loop. Kp~0.070 in duty mode. Stable limit Kp≈0.08.

- D2 — position hold (zone-based). Last working: stopped_vel=0, scale_d=80, max_correction=0.30, max_angle_rate=0.05. Next tuning: max_angle_rate=0.5.

- D3 — steering/yaw

D2 critical fixes applied:

- vel_damp sign was inverted — fixed to +=

- state.theta_ref = -correction (due to enc_pol=-1/motor_pol=-1 config)

## 4.3 pidconfig.txt Format

File: /etc/pidconfig.txt — key=value sections, no positional headers

# pid_config

d1_kp=0.070000 d1_ki=0.0 d1_kd=0.0 d1_ff=0.0

d3_kp=0.0 d3_ki=0.0 d3_kd=0.0

# pos_config

zone_a=8000.0 zone_b=2000.0 stopped_vel=0 scale_d=80

max_correction=0.30 max_angle_rate=0.05 back_to_spot=1

# motor_config

mode=0 pol=−1.0 enc_pol=−1.0 wheel_radius=0.0775

encoder_ppr=145.1 gearbox_ratio=5.2 baud=460800

# system_config

use_batt_adc=0 batt_r1=68000.0 batt_r2=10000.0 batt_trim=1.000000

## 4.4 RoboClaw Infrastructure

E-stop GPIO: 57 (active-low sysfs)

Reset script: roboclaw_reset.py must run before E-stop can be cleared

WriteNVM key: 0xE22EAB7A — causes full config reset on fw v4.2.8. DO NOT USE.

Utilities: roboclaw_util.py (CLI), roboclaw_nc_util.py (ncurses TUI)

## 4.5 WiFi Fix (must reapply after eMMC flash)

DTB patch adds regulator-always-on to /fixedregulator@2 (WLAN_EN = GPIO 105):

sudo fdtput -t s /boot/dtbs/4.19.94-ti-r74/am335x-boneblue.dtb

/fixedregulator@2 regulator-always-on ""

This must be reapplied after any eMMC reflash. Without it WiFi does not work.

## 4.6 New BBB Setup Checklist

- Flash SD card with Debian 12 image

- Put sysconf.txt and TakeFlight_24.psk on FAT32 partition

- Boot (hold button if booting from SD when eMMC has something on it)

- Fold out WiFi antenna

- SSH in: ssh debian@boneblue-0 (password: temppwd)

- sudo apt install -y libncurses5-dev libncursesw5-dev

- pip3 install luma.oled pillow --break-system-packages

- git clone https://github.com/lusher00/balance_bot.git

- cd balance_bot && make && sudo make install

- Apply WiFi DTB fix

- sudo tune2fs -c 1 /dev/mmcblk0p3  (force fsck on boot)

- Always shutdown -h now — never pull power

## 4.7 eMMC Flashing

New Debian 12 image does not include eMMC flasher scripts. Use dd:

sudo dd if=/dev/mmcblk0 of=/dev/mmcblk1 bs=4M status=progress

SD card is 14.8GB, eMMC is 3.6GB — dd will run out of space. Need BeagleBoard official eMMC flasher image or resize partitions first. This is a pending problem.

## 4.8 BBotTuneHUD iOS App

Repo: BBotTuneHUD (SwiftUI). Connects via WebSocket for live telemetry and PID tuning.

Key: armed is operator-only intent (IPC only, never auto-set). trying reflects angle-in-range (auto-set).

## 4.9 Web Dashboard

bbot_dashboard_v4.html — single file, Python HTTP server port 8888. CSV recording with prefixed columns (d1_*, d2_*, d3_*). Encoder counts on Claw tab.

# 5. Raspberry Pi 5 Infrastructure

## 5.1 Hardware

Hostname: rp5-ubuntu (192.168.1.126)

OS: Ubuntu 24

Camera: IMX708 (Camera Module 3) on CAM0

AI accelerator: Hailo-8L

Audio: MAX98357A I2S amp

## 5.2 Camera Config

Ubuntu requires explicit overlay (not camera_auto_detect):

dtoverlay=imx708

camera_auto_detect=0

In /boot/firmware/config.txt

## 5.3 Hailo Cat Tracker

Service: hailo-cat-tracker. Uses IMX708 at 1920x1080 30fps, YOLOv8 inference.

DKMS module rebuild required after kernel updates.

rpicam-vid command that works for autofocus (manual mode locks whatever hardware AF set):

rpicam-vid --codec mjpeg --inline --nopreview --width 1920 --height 1080

--framerate 30 --denoise off --autofocus-mode manual --flush --timeout 0 --output -

## 5.4 Pi 5 OLED

Script: pi5_oled.py. SSD1306 I2C bus 1, addr 0x3C.

Yellow zone: Pi5/Hailo-8 label. Blue zone: IP, CPU temp, Hailo temp, TuneHUD status, time.

# 6. ODrive / ODesc Demo (Pending)

## 6.1 Hardware

ODesc 3.6 clone — runs ODrive 3.6 firmware, identical Python API to ODrive 3.6.

Connected to Pi 5 via USB-C. Photocraft encoder. Beefy brushless motor.

USB udev rule needed:

echo 'SUBSYSTEM=="usb", ATTR{idVendor}=="1209", ATTR{idProduct}=="0d32", MODE="0666"' |

sudo tee /etc/udev/rules.d/50-odrive.rules

sudo udevadm control --reload-rules && sudo udevadm trigger

## 6.2 Gateway Plugin

plugins/odrive_transport.py — uses odrive Python package (pip3 install odrive).

Exposes: pos_estimate, vel_estimate, current_measured, bus_voltage (read-only) and pos_setpoint, vel_setpoint, pos_gain, vel_gain, vel_integrator_gain, vel_limit, current_limit (writable).

Config: configs/odrive_demo.yaml

## 6.3 Demo Vision

Motor + Photocraft encoder + ODesc 3.6 + Pi 5 + TuneHUD dashboard on phone. Cascade position/velocity control. Tune all 6 gains live. Camera shows physical system responding. This is the Hackaday demo.

# 7. Pending Items

## 7.1 Immediate

- Fix MJPEG camera flicker — Flask approach should work but needs testing after latest changes

- Wire ODesc 3.6 + Photocraft encoder + motor to Pi 5

- Test ODrive gateway plugin

- Test BLE transport plugin with Feather

- Get new BBB booted with balance_bot code (deps + clone + build)

- Apply WiFi DTB fix to new BBBs

## 7.2 Near Term

- Record demo video — ESP32 + ODesc + Pi + dashboard on phone

- Write Hackaday article

- Register tunehud.io and closedloopsystems domain

- Create GitHub org closedloopsystems

- Submit TuneHUD_ESP32 to Arduino Library Manager

- Submit to PlatformIO registry

## 7.3 BBB eMMC Flash

New Debian 12 image has no eMMC flasher scripts. BeagleBoard official eMMC flasher image needed. SD card is 14.8GB, eMMC is 3.6GB — cannot use simple dd. Pending resolution.

# 8. Key File Locations

## Pi 5 (192.168.1.126) — ~/TuneHUD/

main.py: Gateway entry point

config.py: Config loader (multi-controller YAML)

camera.py: MJPEG streamer via rpicam-vid subprocess

demo_node.py: Simulated second-order motor velocity PID

plugins/: Transport plugins (ws, modbus, ble, odrive, serial)

configs/: multi_demo.yaml, odrive_demo.yaml, cascade_demo.yaml

tunehud_dashboard.html: Single-file dashboard

start.sh: Startup script

## Mac — ~/Documents/Arduino/libraries/TuneHUD_ESP32/

src/TuneHUD.h: Library header — WiFi + BLE API

src/TuneHUD.cpp: Implementation

examples/BasicPID/: Simple simulated PID

examples/CascadePosition/: Cascade position/velocity controller

library.properties: Arduino Library Manager metadata

## BBB (boneblue-0) — ~/balance_bot/

main.c: Entry point, loads config, starts threads

robot.c: Main control loop, cascade PID

ipc_server.c: WebSocket IPC with dedicated broadcast thread

pid_config.c: Key=value config file parser

telemetry.c: Battery ADC, data logging

/etc/pidconfig.txt: Runtime PID configuration

# 9. Known Issues & Gotchas

- RoboClaw WriteNVM key 0xE22EAB7A causes full config reset on fw v4.2.8 — do not use

- BBB WiFi DTB patch must be reapplied after every eMMC reflash

- websockets v16 changed process_request API — signature is now (connection, request) not (path, headers)

- libcamera Python bindings installed as system package but not in pip Python path — add /usr/lib/aarch64-linux-gnu/python3.12/site-packages to sys.path

- New Debian 12 BBB image boots RO after unclean shutdown — run tune2fs -c 1, always use shutdown -h now

- rpicam-vid autofocus: manual mode with 1920x1080 uses hardware PDAF correctly. Lower resolutions may not trigger PDAF.

- ODesc 3.6 USB may need udev rule to be visible without root

- D2 balance bot: walks/drifts on large disturbances — max_angle_rate needs increase to ~0.5, currently at 0.05

# 10. Development Environment

## Network

WiFi: TakeFlight_24 / FlightSch00!

Pi 5: 192.168.1.126 (rp5)

BBB: 192.168.4.37 (boneblue-0, debian/temppwd)

Feather ESP32: 192.168.1.130

## VS Code Tasks (BBB)

Cmd+Shift+B: rsync + make + make install-only (sync and build default task)

.vscode/tasks.json configured in balance_bot repo

## SSH Config (~/.ssh/config)

Host boneblue-0

HostName 192.168.4.37

User debian

## BBB Shell Setup (~/.bashrc additions)

alias bb="cd ~/balance_bot"

alias jbb="journalctl -fu balance_bot"

alias ll="ls -la"

echo "debian ALL=(ALL) NOPASSWD:ALL" | sudo tee /etc/sudoers.d/debian